Microcontroller Soruları

* = cevap

- = şık

1.Micro işlemcinin içerisinde verinin taşınmasından (Data transmission) sorumlu kimdir?

*Bus Interface Unit

2.Verinin işlenmesinden sorumlu kimdir?

*Execution unit

3.Which of the following flags is set when there is an unsigned overflow?

-Z(Zero)	-C(Carry)	-S(Sign)	-D(Direction)	-O(Overflow)

*C(Carry)

4.Which of the following instructions has something wrong?

-1.mov CX,BL  Wrong

-2.mov BL,CX  Wrong

-3.mov AH,AL

-4.mov CS,CX  Wrong

-5.mov CX,CS

*1,2,4

5.Assume that SP (Stack Pointer) = 100H SS (Stack Segment) = 200H. If push BX instruction is executed BH is copied into _________ linear address.

*20FF

Açıklaması : SP 2100 linear adresini gösteriyor. 2100 e BL yazılıyor sonra SP 1 azaltılıp 20FF’e BH yazılıyor. SP 1 daha azaltılıyor. Stack terse doğru büyüyor!!

5.1.Push BX işlemini yaptıktan sonra SP’nin değeri nedir?

* 0FE

Açıklama : SP herzaman bir sonraki boş yeri gösteriyor.

6.Using little-endian format 4321H is to be stored in an 8 bit memory starting at 200H. What is the content of memory address 201H after storing?

*43

7.1001 0110 0011 BCD to decimal?

*963

8.Which register holds the offset address of the next instruction to be executed?

*Instruction pointer

9.Which register is used to define 16-bit port numbers in I/O instructions?

*DX

10.Assume that CS = 1000H, DS = 2000H, SS = 3000H and IP = 2000H then what is the linear address of the next instruction to be executed by the microprocessor?

*12000H

Açıklama : Instruction pointer sadece code segmenti ile beraber kullanılır. Yani CS*10H + IP.

11.Which of the following segment register cannot be retrieved from the stack?

-1.BS		-2.ES		-3.CS		-4.SS

*CS

12.Suppose that CS = 1200H, DS = 1300H, SS = 1400H, BP = 1500H and		 SI = 100H. Which memory address is accessed by MOV AL,[BP + SI – 200H]?

* 14400H

Açıklama : Memory ile işlem yapıyoruz yani segment olarak DS i seçtik.

Sonuç ise DS*10H + BP + SI - 200H den 14400H çıkıyor.

13.Zero flag is set to _____ if the result of a subtraction is zero and if ____ otherways.

*1	*0

14.ASCII equivalent of a BCD code can be obtained by appending 011 to the left of the BCD code. Base on this information what is the ASCII code of 7 in hexadecimal?

*37H

Açıklama : Kısacası rakamlara 30H decimal ekleyerek ASCII değerini bulabilirsiniz.

15.Which BUS transfers the addres information among microprocessor, I/O devices and memoru?

*Address BUS

16. 0,0390625 ondalık sistemdeki bu sayının hekzadesimal eşiti nedir?

*0,0AH

Kendi çözümüm :

| ÇARPIM | SONUÇ | ONDALIK KISMI |
| --- | --- | --- |
| 0,0390625*16 | 0,625 | 0 |
| 0,625*16 | 10 | A(10) |

0,0AH

17._____ language is constructed using 1’s and 0’s.

*Machine

18. 2KB contains ____ bits.

*2^14 (ama hoca decimal istemiş yani)  16.384 (2*1024*8)

19. Which of the followings are input devices?

-1.Keyboard		-2.Printer		-3.Scanner		-4.Mouse

*1,3,4

20.What is the first microprocessor?

*Intel 4004

21.What is the first calculator?

*Abacus

22.What is the first mechanical calculator?

*Abacus

23.Which field of an instruction specifies the operation to be performed?

-R/M		-MOD		-Direction		-REG		-Opcode

*Opcode

24.Which of the following is the correct order of the basic instruction cycle?

*Fetch, decode, execute

!!! Not : Bu kodun çıktısı ne olur, bu kod çalıştıktan sonra şu registerın içeriği ne olur gibi sorularda olabilir.

Dersten notlar(Boş bazı notlar okumayabilirsiniz) :

Önemli insanlar

- Ada Byron? (Charles Babbage ile analitik makinenin geliştirilmesinde çalışıyor)

- Charles Babbage

- Jon Von Neumann (Programları memory’e yazmayı akıl eden ilk kişi)

- Alan Turing (Almanların şifresini çözen Colossus bilgisayarını yapan kişi ve Alan Turing testi)

- Jeff Lenington (Nöral networklerle ilgili çok çalışma yaptı, Makine öğrenmesinin adımlarını attı)

- Paul Allen

- John McCarty